# Project Capture Support Tool: Observation Equipment,

# GoPro Hero 4

Author: M. Smits

Date: 02-2017

Version I

# Introduction

This support tool guide will help you to prepare for your project in Kenya. As you are designing and building a dwelling for a rural family it is of vital importance to capture the progress you make over the months. In the rural housing studio you will use a GoPro Hero 4 to record your project. In the image below a typical rural family compound is shown. A family, which you will help to, built a new house, currently lives in a dwelling like this. Before you can start, you will be involved in talking to the family and map their compound out. All of these processes are vital to capture. In this way every single ‘action’ you take will be registered and can be analysed afterwards.

# Aim

In your project the GoPro (Hero 4) will act as a static 'macro' (long distance) observation point, which will be permanently positioned throughout the project on one spot, mounted on the large tripod (hama, star 63). From this position the GoPro will be solely used to create a time-lapse of the building process. This guide will show you in easy steps how to register and use the different recording methods/items to produce a clear recording for the 4-5 months showing that you are designing and constructing the family dwelling.

# Index

# Observation Pilot I

In order to prepare for your time in Kenya, pilot(s) are organized in the months previous to your departure. Some of the pilots are individual and some are in groups. This has no influence on the team composition in Kenya. Here for sure you will work in duo’s (1 student & 1 architect). With these pilots and support you will know what to do in the field (Kenya) and you will be able to focus on getting all the information which is needed for the outcomes to the project. The main aim of the TimeLapse observation is to compare how you organise and build the new house for the family.

This pilot will help you to practice with the Gopro Hero 4 to make a ‘macro’ TimeLapse observation by the following steps.

- Inventory

- Settings GoPro

- Recording a static object with making processes

- Assignment

- Mandatory settings GoPro in Kenya

- Mandatory points using GoPro in Kenya

In step 1, 2 & 3 the various settings and usage of the GoPro for TimeLapse observation will be explained. Take time to go through the various instruction movies to make a TimeLapse observation. In step 4 you will work in duo’s to prevent too much individual work.

## Step 1: Inventory

Before starting the research it will be necessary to check if the required equipment is available to be used and properly prepared (charged batteries and camera settings). You should check this the night before so everything is set for the next day.

In the Appendix 1 you will find: Step 1_Checklist, this list will help you prepare for the pilot project for the next day.

Evening Check:

The first two items are to be checked the night before the observation. Which means you will have fully charged GoPro batteries and solarcharger. In case the GoPro batteries run out, the solar charger will recharge one battery in about 4 hours.

Morning Check:

On the morning of the observation use the same checklist as shown in Appendix one. Every time you put something in the bag check the item on the list (item 3 till 9). The picture on the right shows the whole equipment necessary for the recordings of the pilot project.

## Step 2: Required Settings: GoPro

A GoPro Hero 4 will be used for the production of a time-lapse of the design and construction process of a house in Kenya. The following step will explain the main settings, which need to be taken into account making a time-lapse.

The camera has a very user-friendly manual from the manufacturer, it can be found in the box below the camera. Most of this information will be an interpretation of the user-manual/online tutorials of the manufacturers and personal experience. So for the basic information of the equipment or various conditional difficulties, it is recommend to use the manual.

This guide developed for the observations will only explain the following required settings:

- Battery life

- Field of view/ photo resolutions

- Frame per seconds

- Advanced setup setting such as ISO-limits

### 1. Battery life

When you are in the field battery life is one of the most important considerations. Without the battery the camera will obviously stop recording and you will lose important information. In the field you will participate for 4-5 months in a design and building activity. To keep best track of the progress from a static point of view, the Time Lapse function of the GoPro is being used. This maximizes battery length and reduces the amount of footage to a bare minimum. For this reason the whole guide focuses on explaining the Time Lapse function. In a moderate climate like The Netherlands the battery can easily last up to 4 hours in time-lapse mode. Exposed to the burning sun on Mt Elgon this time can be reduced to 2-3 hours of battery life. There are five important ways to maximize the battery life:

- Keep wifi off

- Keep large display off

- Use low resolution

- Use the right casing

- Keep camera out of the sun

1. Keep wifi off: Although the wifi connectivity is really useful to track recordings of the GoPro, it drains the battery very fast. Therefore the wifi should be kept off at all times! In the following link you will find an instruction video how to put the wifi on the GoPro off: link

https://drive.google.com/open?id=0Byeg_wxkIbwtbmZrTDZvLXhmTTg

2. Keep large display off: The touchscreen of the GoPro is great feature to check the timelapse pictures  being made. However, the screen also drains the battery very fast. Therefore the screen should be turned off at all times. In the following link you will find an instruction video how to put the wifi on the GoPro off: link

https://drive.google.com/open?id=0Byeg_wxkIbwtQXh5eE5TWElGX0U

3. Use low resolution & long interval: The GoPro has four different Mega Pixel (MP) settings for the Timelapse function.:

- 12mp wide

- 7mp wide

- 7mp medium

- 5mp medium

The higher the MP you use, the more battery it will take. However, lower the MP will create less detailed pictures. Changing from a wide angle to a medium angle will make the observed field smaller. For this reason we advise to use the 7mp wide setting.

Another important aspect is the interval time of the picture taken. Smaller interval times (0,5 sec-5 sec) means more pictures per minute. Bigger interval times (10 sec-60 sec) means less pictures per minute. The less pictures you take the longer the battery will last. Therefore a longer interval time is advised. In the following video how to change both resolution & interval for the time lapse are shown: link

https://drive.google.com/open?id=0Byeg_wxkIbwtWHNWUVFLNWxCUE0

4.Use the right casing: The GoPro standard comes with a waterproof casing. This protects the GoPro against merely everything. Sadly it does not protect against the sun.

When you use the closed back panel shown in the image below. It will make the GoPro watertight, however, the GoPro can’t ventilate its heath. This will drastically drain the battery and will reduced it to an operating time of 2-3 hours (instead of 4)

When you use the open back panel shown in the image below. It will open the GoPro and enable it to ventilate its heath. This will improve the battery life span to 3,5-4 hours. However, the Gopro is now exposed to rain and dust! Make sure whenever the rain start you change the case or make a small roof over the Gopro

5.Keep camera out of the sun: To further protect the GoPro against overheating and rain it is advised to make an improvised ‘hat’. In the example below a hat was literally used to protect the GoPro, obviously other more structural protection that is stable enough against prevailing winds, birds and animals is advised. It is up to yourself to make such a ‘shelter’ on site in Kenya later on.

### 2. Field of view

Field of view also known as FoV, determines how much of the area will be captured on the time-lapse pictures. FoV consists of three elements: Mega Pixel, Distance and Angle. This can be altered in the settings of the Time-lapse mode. As explained in the previous section the Hero 4 silver has 4 different settings for photo resolutions, containing:

- 12mp wide

- 7mp wide

- 7mp medium

- 5mp medium

Mega Pixel

However only the wide or medium part will be important to determine the FoV. The difference between MP (mega pixels) will only affect the quality of the taken pictures.. The highest MP on the GoPro Hero 4 is 12 mp wide.

Distance

In the images below an example observation of a lady reading a book is given. In the first image a map of the location is given. The green dot is the target location and the red dots are the various observation distances used in this observation (5/10/15/20 meter).

Just like your future observations in Kenya it is important to make a well-balanced decision which distance you will take for the permanent (static) observation with the GoPro.

Below you will find an example project with the use of the different settings on various distances, in order to capture the woman reading a book in a small park.

We started by measuring and marking the distances to the observation location (in this case the steel railing in the back). I used branches to mark the distances of 5, 10, 15 and 20 meters.

To give you an idea of what the initial distance does to the observed topic an overview is given below. You will notice that however you get to observe the topic (woman reading book) with greater detail, you will loose more of the context. Where in the 20 meter distance you can see most of the shops most, in the 2 meter picture you lost this context. Other aspects: continuation of the trees on both sides of the observed topic, parked cars and house entrances, are also lost. We also call these aspects: Macro Processes.

20 meter

15 meter

10 meter

5 meter

2 meter

However, a lot of the context perspective is lost, the observed topic can be described more accurately: What she wears, facial expressions, holding the book with both hands, etc. We also call these aspects: Micro Processes. The aim of the right static observation point is to mediate between how much details you need to see of the observed topic and how much of the context is important. Which in this case is the 10 meter picture; here you can see just enough of the context and are also just able to observe the facial expression.

In the following example we compare two observations:

- 7mp medium, 1 second interval 20 meter: link

https://drive.google.com/open?id=0Byeg_wxkIbwta3EzYTM5NHVzR3M

- 7mp medium, 1 second interval 5 meters: link

https://drive.google.com/open?id=0Byeg_wxkIbwtRWhkRWFXTFdkcFU

In the first movie there is a lot of distance (20 meter) to the topic. Although it has weaknesses (for example: less detail of observed subject), it allows the subject to behave naturally in its environment. In this case the lady with the book can be observed without invading her privacy and with little distraction.

In the second movie the shortness of distance (5 meter) creates a more intrusive observation. Because you as observed get so close to the subject you are distracting the subject in its natural behaviour. Because of that you are altering the behaviour of the subject and thus influence the observation. In this movie you can also see a man observing the woman from even closer. Although he is able to observe many details (page number, clothing brand, shoe size, etc.) he is clearly distracting the subject in its natural behaviour. Again make sure you keep sufficient distance not to distract the subject, however, close enough to see the needed details. Which in this case: which book she reads, how she reads it, if she engages with the context and how she does that.

In Kenya you will need to observe, how the design and construction is being realized including all the sides processes! For example: you want to observe how the new house is being constructed, however, you would also like to know how a carpenter makes the front door.

Angle

The GoPro has two types of angle setting: medium and wide. The medium lens setting has a more zoomed-in/ close up look of the situation, as the wide lens will capture a broader area. Even on 5m distance of the subject it is still possible to capture the whole subject and a lot of the context.

7 MP, 20 meter Medium

12 MP, 20 meter Wide

As the wide lens will be ideal for an overall view of the situation, you might want to include more of the surroundings. The medium lens will be useful for more focused shots of the situations, only downside will be the lower quality of pictures. Although this will help to extend the battery life.

## 3. Frames per second

One of the most important settings will be the interval setting this alters the amount of pictures/frames per second taken. The GoPro Hero 4 has several options ranging from a frame per 0.5 second up to a frame per 60 seconds. According to the manufacturer's manual the following advice has been given for the various interval settings:

- 0.5 – 2 seconds: surfing, biking or other sport

- 2 seconds: busy street corner

- 5 – 10 seconds: sunset

- 10 – 60 seconds: lengthy activities such as construction projects (to be used during the field experiment)

Obviously the shorter the duration between frames the more detailed actions can be captured. In particular with very fast processes (like biking, surfing, etc.), as these actions will be done in less than a couple of seconds. On the contrary slow operations (building a house) will have a better quality of recording when longer durations between frames are being used. In these slower processes actions might even take up to an hour before having any noticeable difference in the process.

Choosing the right setting needs to be determined by what do you require to record during the process.

- If you only need to capture a long transformation process of a static object, 60 seconds between pictures will be fine.

- If the time-lapse requires to capture the process involved in the transformation 10-30 seconds will be increase insight substantially.

- If you would like to capture the interaction between the participants during this transformation process of the object, 2-5 seconds will be necessary.

In this first example we go back observing the woman reading a book. Here we use the 20-meter distance 7mp and a 1 sec interval, during 34 frames (observed length 34 seconds): link

https://drive.google.com/open?id=0Byeg_wxkIbwta3EzYTM5NHVzR3M

And compare the same observation using the 20-meter distance 7mp and now a 30 sec interval, during 2 frames (observed length 34 seconds): link

https://drive.google.com/open?id=0Byeg_wxkIbwtQkFpOGd6ejdvWEU

A completely different observation is being made of the same process. Cutting out the whole process of walking, making weird movements, people passing by, etc. Before you start the observation, think of the topic you want to observe and how important surrounding subjects and processes are to your observation.

A similar situation will occur during your project. Here you should not only focus on what needs to be observed now but more importantly what needs to be observed in the future. In the image below a possible future house is placed in the same observation point.

You might have captured the processes before the construction phase and preparation processes (lady carrying things; in the right). However, you will have captured half of the building (on the left). Planning the permanent observation point is essential. In the image below sufficient distance and the right angle are used to observe all processes of the project.

Another important aspect will be the marking of your observation point. You will need to find the right angle, height and distance, for over 4-5 months in the field. Make sure you use small objects that can not be (re)moved by children or animals. We suggest to use small stones that you partially bury in the sand.

In this way you will be able to every day find your observation point back. Put the tripod in place and mount the GoPro in the same direction.

## Light Exposure

The default settings of a gopro camera are quite light sensitive in particular for artificial light sources. This could changed by enabling the spot-meter. Which will help to tune down the light exposure if the camera is taking pictures from an illuminated space while being positioned in a darker space. In the following movie we explain how both the Spot-meter as Protune (explained below) are put on or off: link

https://drive.google.com/open?id=0Byeg_wxkIbwta1l1Sy1RQjhpV1k

If the recordings still suffers from light exposure, turn on Protune in the time-lapse settings, this enables a lot of extra settings to adjust the brightness and quality of your pictures.

As lowering the ISO limits the defaults limits will be 800, which creates brighter photos in very low light. Besides lowering the exposure it also helps to lower the amount of image noise in the taken pictures. The 800 ISO limit also has an increased image noise, the limit can be lowered up to 100, which will create a darker photo indoor lighting with minimal image noise.

Another setting in Protune called EV compensation (Exposure value compensation) will also help to lower the light exposure.  This setting adjusts brightness within the existing ISO limits.

The last setting in the Protune mode which adjust the exposure is white-balance. Even though the default setting will auto-adjust the balance while according to the environment. It might be useful to alternate the brightness by changing it manually.

In the case you start early and finish late there might be a lack of light exposure. If you are not able to adjust the brightness by using one of the Protune settings. We would suggest changing the time-lapse modus to night-lapse modus. This will enable to take much brighter recordings.

- Recording a static object with making processes

- Advised settings GoPro in Kenya

## Step 3: Recording a static object with making processes

After having clarified the important settings/modus for using the GoPro camera. It is now up to you to start recording an observation of a developing/making process.

As already has been described in step two, settings as MP and interval will be important to capture a clear time lapse to determine which process of the project you want to highlight.

- Is the interaction between participants important?

  - Or even the interaction with the surroundings?

- Are the general construction proceedings important?

  - Only of a specific set or even its surroundings?

- Or just the transformation of the static object important?

Before starting the observation it is important to sketch make an observation plan. In the map you sketch the situation, with the following elements:

- Where the subject is located (lady reading a book)

- What you want to know of the subject: what she reads, how she reads and how people react in her surrounding.

- How are you going to observe the subject: I chose 4 different distances to find what the ideal distance is to observe the above elements I want to know. I used the distances 20, 15, 10 and 5 meter.

During the test recordings it is important to register which elements could/should be observed and adjust the observation point accordingly.

- After 4 test rounds (marked on the map) on the distances 20, 15, 10 and 5 meter (1,5 minute each) I concluded that the 10-meter observation was ideal.

- To observe the lady reading, I directly started to record her, the ideal distance (10 meter) was set. In the end she was reading for 15 minutes and then she left.

Coding

After your observation it is time to retrieve everything from your GoPro. The first step is to transport the pictures to the Seagate drive you have received (2 TB). You will need to make separate folders of every day you are making the observation on Mount Elgon. The name of the folder should be setup the by following example:

T01.PR.2017.10.18 (Team 01.PelleRademakers.year.month.day)

Team numbers are:

- Pelle Rademakers

- Maud van Dijck

- Damian van der Velden

- Atdhe Lila

We advise you to install Gopro quick: link. This GoPro application will help you to transfer pictures easily from the Gopro to your computer via the USB cable.

Finally it will be important to use a specific code for all recorded media inside the destination folder (as explained above), to have a well organised hard drive. We will use a specific coding system to easily keep track of your data, this will be: Team number/initials observer/kind of recording/construction phase/date/ media number.

- Team number: T(number)

- Initials observer: first letter of first name and surname

- Kind of recording:

  - Time-lapse: TL

  - Close-up: CU

  - …

- Phase: P(number)

  - Phase 1: First introduction family

  - Phase 2: Program of demands

  - Phase 3: Design brief

  - Etc.

- Date: European calendar system

- Media number: This will be given automatically when renaming a bash of media

An example of the code: T1.PR.TL.P1.2017.10.18.1

Below an example how your folder structure should look like.

# Step 4: Assignment

During this step you will start with your first recording in the field (in duo’s). You will record a building/developing/making process of your own choosing. You are asked to make two observations; one short (micro observation) & one long (macro observation)

1. A short (micro observation): Can be someone cooking a dish, sweeping the garden, etc. This observed subject should be between 2-30 minutes long. The observation should focus on micro processes, such as: getting materials, cutting, etc.

2. A long (macro observation): Can be a student making a graduation 3d model, painting of a fence, etc. This observed process should be between 7-9 hours long. The observation should focus on macro processes, such as: people moving through an area, hand or facial gestures, etc.

In both observations you should repeat the previously explained steps and apply this on your new situation:

- Step 1: Is your equipment ready?

- Step 2: Do you have the right: setting/mode/distance/height/etc.?

- Step 3: Did you make an observation plan? Did you adjust distance and height according to the subject? Did make separate folders? Did you use the right coding? etc.

Final Assignment

The final product of the assignment should be an tutorial movie where you explain the viewer all the observation steps you made in preparing, executing, coding, etc.

- If you find any difficulties in reading/understanding this support tool please use the google doc and change the editing mode in the right top corner to suggesting.

- If you find any difficulties in executing the observations, clearly explain in the video how you tackled them and suggest in the guide what you would change/add.

To make this movie you should use the Lumix Camera. In this movie you will use your voice to explain everything (so not as in our example of the lady reading; with subtitles). Also explain which micro and macro processes you observed.

Example of a good video tutorial:

https://www.youtube.com/watch?v=pPPDOM4nbnE

Software you can use for video editing:

- Windows Moviemaker (free: novice level)

- Adobe premiere (10,20 euro, Hbo Plaza: Expert level)

Software you can use for screen capturing:

http://screencapturer.com/

When finished

- Upload the results to your groups Googledrive (invites have been sent) folder and share with the other team.

- View each others movie and give feedback

- Make adjustments according to each other's feedback

- Make a final version and put in your Googledrive

# Step 5: Mandatory settings GoPro in Kenya

- Keep wifi off

- Keep large display off

- Use resolution: 7mp wide

- Keep Light-Point: on

- Keep Protune: off

- Keep camera out of the sun

- Use the right casing:

  - Closed back panel during rain

  - Open back panel during sun

# Step 6: Mandatory points using the GoPro in Kenya

- Make sure you walk through step 1 to 3 upon arrival in Kenya

- Make sure you go through the checklist every evening and morning.

- Make sure the tripod is standing every day on exactly the same position

- Make sure the camera has the same setting every day

- Make sure the camera has the same orientation every day.

# Appendix 1

|  | Step1: Checklist | Date | ?????? |
| --- | --- | --- | --- |
|  |  | Daynumber: | 1 |
| Evening (before) Checklist |  |  |  |
| 1 | Gopro batteries (2x) charged |  |  |
| 2 | Solar charger: charged |  |  |
| Morning Checklist |  |  |  |
| 3 | Gopro camera |  |  |
| 4 | Gopro batteries (2x) |  |  |
| 4 | Solar charger |  |  |
| 5 | Dual Charger with cable |  |  |
| 7 | SD memory card |  |  |
| 8 | Hama star 63 tripod |  |  |
| 9 | Gopro in the right cover |  |  |